D1.1 - Use Cases Description

Overview of use case

UC4: Critical electricity distribution grid operation in energy communities

Surrounding ecosystem: UC4 is related to the energy sector, with focus on energy communities. It contributes to the technical end economic operation of the electricity distribution grid.

Potential contribution area in the port environment: It will improve the operation of the port distribution grid. Artificial intelligence will be used to handle energy data.

Potential contribution area into the Sa4CPS concept: This UC provides situational awareness of the electricity consumers in order to rise their participation in demand response programs.

| Description of the scenario | A distribution network operated by an energy community aggregator, as a cooperative, wants to develop the means to use flexibility in case of reliability issues in the distribution grid or when asked by the upstream network operator. At the same time, this operator is managing an energy community composed by a part of the consumers in the area, targeting to optimize the energy contracts definition considering user’s flexibility, energy prices, and operation costs. A flexibility aggregator gets involved to collect and provide this flexibility. |
| --- | --- |
| Value chain roles | The use case includes: - Consumer - Community Manager - DSO - Flexibility Provider/Aggregator |
| Targeted market | - Distribution system operation - Energy communities - Electricity consumers participating in energy communities |
| Targeted business impact | - Use of flexibility as an asset for power system operation - Optimization of resource use for minimizing costs |
| Targeted innovation | - Use of Explainable AI (XAI) for situational awareness - Activation of Flexibility with XAI - Use of federated learning models for energy communities |
| Challenges focused | Distributed AI/Explainable AI Digital Twin Consumer involvement |

Use Case Description

A distribution network operated by an energy community aggregator, as a cooperative, wants to develop the means to use flexibility in case of reliability issues in the distribution grid or when asked by the upstream network operator. At the same time, this operator is managing an energy community composed of a part of the consumers in the area, targeting to optimize the energy contracts definition considering user’s flexibility, energy prices, and operation costs. A flexibility aggregator gets involved to collect and provide this flexibility.

Identification and description of scenarios

| Scenario 1: Voltage Management | DSO detects voltage in the grid out of the normal range. Then asks the aggregator of flexibility to provide load management in the affected branch of the grid. |
| --- | --- |
| Scenario 2: Line Capacity Management | DSO detects power flow in a cable higher than its capacity. Then asks the aggregator of flexibility to provide load reduction in the affected branch of the grid. |
| Scenario 3: Economic operation of communities | Community operator asks flexibility to, for example, keep the community self-sufficient during certain hours. It may happen because of high dynamic prices of electricity. |

Identification of technological challenges

DR can provide cheap and reliable AS that were exclusively provided by generators, as well as other consumer-based solutions that can help to reduce market power. When flexibility needs arise due to RES variability, thanks to the advances in Information and Communication Technologies (ICT), DR counts as necessary infrastructure to fully participate in the system flexibility throughout DR products. However, for the DSO to directly manage flexible loads or to provide DR programs to electricity end-users, many technological and legal restrictions need to be overcome.

Main contribution area to the Sa4CPs concept: Distributed AI/explainable AI, digital twin. Several initiatives in the field focus on energy aspect, ignoring the learning of user's comfort and flexibility.

Targeted innovation boosted by the Sa4CPS concept: Use of Explainable AI (XAI) for situational awareness; Activation of Flexibility with XAI; Use of federated learning models for energy communities

Target business impact: Use of flexibility; Optimization of resource use

Contribution areas of partners

| CEL | - Facilitate integration in the field - Act as energy community manager providing knowledge and requirements to the project |
| --- | --- |
| Digitalmente | - Monitoring and actuation of several appliances and grid assets. - Integration of IoT devices and derived actuation platform. - Local management (fog-computing). - Energy community management platform. |
| Enerim | - Provide knowledge to a flexibility services provider (to interface via REST Api) |
| ISEP | - Conceive and develop energy forecasting algorithms for individual energy loads and resources - Conceive and develop an algorithm for the definition of single appliances shifting possibilities - Conceive and develop an energy management solution based on optimization algorithms for energy cost minimization and users’ comfort maximization. |

Roadmap for development

Year 1:

- Define use case scenario(s)

- Define concept requirements

- Early tech demos

Year 2:

- Architecture design and prototyping

Year 3:

- Pilot implementation in overall concept

KPIs of the UC

| KPI | Initial Value | Targeted Value |
| --- | --- | --- |
| Number of consumers enrolled in flexibility program | 0 | 10 |
| Amount of Energy transacted (kWh) | 0 | 20 |
| Voltage issue events number | 20 | 2 |
| Average duration of voltage issues (min) | 30 | 10 |
| Average time required to solve voltage issues (min) | NA | 10 |
| Line capacity issue events number | 45 | 5 |
| Average duration of line capacity issues (min) | 45 | 10 |
| Average time required to solve line capacity issues | NA | 10 |

D1.2 - Requirements

Descriptions of the use case surrounding ecosystem

UC4 focuses on the secure and efficient operation of critical electricity distribution within the context of an energy community. The ecosystem involves multiple stakeholders collaborating to manage local energy resources, enhance grid flexibility, and ensure reliable power delivery while prioritizing consumer trust, security, and privacy. Key stakeholders include:

- Energy Community Members (Consumers): End-users participating in the community, sharing data (with consent), potentially offering flexibility from their assets (e.g., solar PV, batteries, controllable loads), and benefiting from community energy management.

- Energy Community Manager: Acts as the central coordinator, interfacing directly with consumers, managing community operations, aggregating flexibility and ensuring regulatory compliance (including GDPR and energy sector regulations).

- Technology Providers: Supply the core technological infrastructure, including the energy community management platform, monitoring systems for consumer appliances and grid points, IoT device integration, actuation capabilities, and local data processing (fog computing) solutions.

- Distribution System Operator: Responsible for the local distribution grid. The DSO is a key stakeholder interacting with the energy community, utilizing its flexibility services for grid balancing and stability and setting grid connection/operational requirements.

- IoT Device/Hardware Providers: Supply the physical sensors, meters, and actuators deployed at consumer premises and grid locations.

- Regulatory Bodies: National (e.g., ERSE in Portugal) and European authorities setting the rules for energy communities, data protection (GDPR), cybersecurity, and grid operation.

The ecosystem aims to create a localized energy system where consumers actively participate, managed transparently by the community manager and enabled by secure, privacy-preserving technology, ultimately contributing to a more resilient and efficient distribution grid.

Value Chains

The value chain in UC4 is collaborative, revolving around data exchange, energy flexibility, and trust management:

- Consumers to Community: Provide consented data and energy flexibility (demand response, generation shifting). Receive value through potential cost savings, participation in a sustainable initiative, and reliable energy service.

- Community Manager to Consumers: Provides management services, transparency, ensures privacy/security compliance, builds trust, and facilitates participation benefits.

- Technology Provider to Community Manager/Community: Delivers the operational platform, monitoring/control tools, IoT integration, and secure data handling infrastructure (including fog computing). Enables the community's functions and implements necessary TSP features.

- DSO to Community: Provides the physical grid infrastructure and potentially receives value from grid services offered by the community, potentially deferring traditional grid reinforcements.

- IoT Providers to Technology Provider/Community Manager: Supply the necessary hardware components for monitoring and control.

Trust, enabled by robust security and privacy measures implemented by Digitalmente and managed transparently by CEL, underpins the entire value chain, facilitating data sharing and consumer participation essential for flexibility services.

Market state of art

The energy sector is undergoing a significant transformation driven by decarbonization, decentralization, and digitalization. Energy communities are emerging as key organizational models to integrate distributed energy resources (DERs) and engage consumers.

- Trends: Growth in local energy initiatives, smart grid deployments, increasing adoption of DERs (solar PV, batteries, EVs), development of flexibility markets, and a heightened focus on cybersecurity for critical energy infrastructure.

- Technologies: Advanced Metering Infrastructure (AMI), IoT sensors for real- time monitoring and control, Energy Management Systems (EMS) platforms (often cloud-based), fog/edge computing for local optimization and privacy enhancement, secure communication protocols (TLS/DTLS), and increasing exploration of blockchain for peer-to-peer trading and data integrity.

- Practices: Demand-side management programs, aggregation of DERs for grid services, adherence to grid codes, implementation of cybersecurity frameworks, strong emphasis on data privacy compliance (GDPR), and developing consumer-centric service models. Secure software development (SSDLC) and robust authentication/authorization (OAuth 2.0, OIDC, RBAC) are expected for platforms managing sensitive energy data and control functions.

- Challenges: Ensuring interoperability between diverse devices and platforms, establishing clear regulatory frameworks for data sharing and flexibility markets, scaling solutions cost-effectively, maintaining consumer trust and engagement, and protecting critical systems against evolving cyber threats.

UC4 addresses these state-of-the-art practices and challenges by developing and demonstrating a TSP-centric energy community model. It integrates diverse technologies (IoT, fog, platform) with a strong focus on secure communication (MQTTs, HTTPS), robust platform security (OWASP), and technical implementations adhering to industry security guidelines, aligning with the requirements of critical infrastructure operation and GDPR.

| ID | Name |
| --- | --- |
| REQ_UC4.9 | Energy Community Setup |
| REQ_UC4.10 | IoT Data Retrieval |
| REQ_UC4.11 | KPI information |
| REQ_UC4.12 | DSO signal of flexibility |
| REQ_UC4.13 | Energy Community Overview |

D2.1 - Technological requirements architecture

Requirements

| ID | Name | Requirement Category | Description | Expected Result | Author |
| --- | --- | --- | --- | --- | --- |
| REQ_UC4.1 | Missing data replacement | Product | Missing data needs to be replaced with, e.g. baseline data | 50% accuracy | ISEP |
| REQ_UC4.2 | Data availability | System | 15-minute energy usage data needs to be available on time | Hard limit = xx seconds / minutes. | ISEP |
| REQ_UC4.9 | Energy Community Setup | Software | Allow for in-platform community creation | In-platform regulatory guides and process submition tools | Digitalmente |
| REQ_UC4.10 | IoT Data Retrieval | System | Integration of IoT devices | 15-minute compliency at all levels. The right information at the right time also for the end customer | ISEP |
| REQ_UC4.11 | KPI information | System | The KPI related to energy aspects to the user to be updated regularly | 1 min at local level information; 15 min at other perameters | ISEP |
| REQ_UC4.13 | Energy Community Overview | Software | Comunity data presentation, including consumption and production information by consumer / prosumer | In-platform visual overview of relevant datapoints | Digitalmente |
| REQ_UC4.14 | Energy Community Import / Export | Software | Allow for the visualization of grid consumption and exports | In-platform visual overview of relevant datapoints | Digitalmente |

D3.1 - Technological requirements for perception and comprehension

| ID | REQ_UC4.1 |
| --- | --- |
| Name | Missing data replacement |
| Requirement Type | Product |
| Related Task (T3.1-T3.4) | T3.1, T3.3 |
| Description | Missing data needs to be replaced with, e.g. baseline data |
| Expected Result | Gaps in time-series energy data are filled using a defined method (e.g., baseline values). |
| Author | ISEP |

| ID | REQ_UC4.2 |
| --- | --- |
| Name | Data availability |
| Requirement Type | System |
| Related Task (T3.1-T3.4) | T3.1, T3.3 |
| Description | 15-minute energy usage data need to be available on time. Hard limit = xx seconds / minutes. |
| Expected Result | Processed 15-minute energy data is available within a specified time limit (limit TBD) after the interval ends. |
| Author | ISEP |

| ID | REQ_UC4.3 |
| --- | --- |
| Name | Data storage security for sensitive data |
| Requirement Type | Software |
| Related Task (T3.1-T3.4) | T3.1, T3.3, T3.4 |
| Description | Data to be validated and encrypted or otherwise protected within databases. Validations centralized and synchronized and selected data in database has additional protection. |
| Expected Result | Sensitive data in databases is validated upon entry and protected (e.g., encrypted). Validation logic is centralized. |
| Author | Enerim |

| ID | REQ_UC4.4 |
| --- | --- |
| Name | Database connections everywhere encrypted |
| Requirement Type | Software |
| Related Task (T3.1-T3.4) | T3.1, T3.3, T3.4 |
| Description | Database connections are always encrypted, and data is sent in a secure manner. TLS in use for database traffic. |
| Expected Result | All connections to/from databases utilize TLS encryption for data in transit security. |
| Author | Enerim |

| ID | REQ_UC4.5 |
| --- | --- |
| Name | Protected data management via REST API calls |
| Requirement Type | Software |
| Related Task (T3.1-T3.4) | T3.1, T3.3, T3.4 |
| Description | REST API to support data protection and return only allowed data content through API call. Selected data has additional protection when called through REST API and REST API answers to incoming business needs. |
| Expected Result | REST APIs enforce data access controls, return only authorized data, apply additional protection to sensitive data, and meet business needs. |
| Author | Enerim |

| ID | REQ_UC4.6 |
| --- | --- |
| Name | User Management Improvements |
| Requirement Type | Software |
| Related Task (T3.1-T3.4) | T3.3, T3.4 |
| Description | User roles and user groups to support detailed access management. New authentication solution with full features into use for product. Authentication and authorization answers to additional business needs and is centralized with additional features. |
| Expected Result | System implements enhanced, centralized user role/group management and a new authentication/authorization solution meeting business need for access control. |
| Author | Enerim |

| ID | REQ_UC4.7 |
| --- | --- |
| Name | Energy Community management |
| Requirement Type | Software |
| Related Task (T3.1-T3.4) | T3.3, T3.4 |
| Description | UI improvements and REST API calls to support Energy Community management. Development of new features and improvements for handling of Energy Communities. Validating the full potential of energy community services. |
| Expected Result | The platform provides improved UI elements and APIs for managing Energy Communities, including new features to enhance their functionality. |
| Author | Enerim |

| ID | REQ_UC4.8 |
| --- | --- |
| Name | Data availability (End User) |
| Requirement Type | Software |
| Related Task (T3.1-T3.4) | T3.1, T3.3 |
| Description | 15-minute energy usage data needs to be available for the end user on time. 15-minute compliancy at all levels. The right information at the right time also for the end customer. |
| Expected Result | End users can promptly access their compliant 15-minute energy usage data through the platform's interfaces. |
| Author | Enerim |

| ID | REQ_UC4.9 |
| --- | --- |
| Name | Energy Community Setup |
| Requirement Type | Software |
| Related Task (T3.1-T3.4) | T3.3, T3.4 |
| Description | Allow for in-platform community creation. In-platform regulatory guides and process submission tools. |
| Expected Result | Users can successfully create energy communities via the platform, access relevant guides, and use submission tools. |
| Author | Enerim |

| ID | REQ_UC4.10 |
| --- | --- |
| Name | IoT Data Retrieval |
| Requirement Type | Hardware |
| Related Task (T3.1-T3.4) | T3.1, T3.3 |
| Description | Allow for the retrieval of data points from an array of IoT devices, installed in community participating installations. |
| Expected Result | The system can successfully connect to specified IoT devices and retrieve relevant energy data points. |
| Author | Digitalmente |

| ID | REQ_UC4.11 |
| --- | --- |
| Name | KPI information |
| Requirement Type | System |
| Related Task (T3.1-T3.4) | T3.1, T3.3 |
| Description | The KPI related to energy aspects to the user to be updated regularly (1 min at local level information; 15 min at other parameters). |
| Expected Result | Users are presented with relevant energy KPIs, updated at specified frequencies (e.g., 1 min for local, 15 min for others). |
| Author | ISEP |

| ID | REQ_UC4.12 |
| --- | --- |
| Name | DSO signal of flexibility |
| Requirement Type | Services |
| Related Task (T3.1-T3.4) | T3.1, T3.3, 3.4 |
| Description | The DSO to be able to control emergency flexibility. Direct load control for some loads in real-time. |
| Expected Result | The system can receive signals from the DSO and execute emergency flexibility actions, including real-time direct load control. |
| Author | ISEP |

| ID | REQ_UC4.13 |
| --- | --- |
| Name | Energy Community Overview |
| Requirement Type | Software |
| Related Task (T3.1-T3.4) | T3.1, T3.3, T3.4 |
| Description | Community data presentation, including consumption and production information by consumer / prosumer. In-platform visual overview of relevant datapoints. |
| Expected Result | Users can view a visual overview of the energy community's data, including consumption/production breakdowns per member. |
| Author | Digitalmente |

| ID | REQ_UC4.14 |
| --- | --- |
| Name | Energy Community Import / Export |
| Requirement Type | Software |
| Related Task (T3.1-T3.4) | T3.1, T3.3, T3.4 |
| Description | Allow for the visualization of grid consumption and exports. In-platform visual overview of relevant datapoints. |
| Expected Result | Users can view a visual overview of the energy community's energy import from and export to the grid via the platform. |
| Author | Digitalmente |

D5.1 - UC4 risks, challenges and requirements

Risks and challenges

- Special protection for sensitive data - Specific protection of data in encrypted or otherwise protected databases. Data processing only by authorised users of sensitive data.

- Identification of the authorised user. Creating and managing the encryption.

- Task 5.1 and 5.2

- Encrypted database connections - Data is secured while sent between systems example via API calls.

- Data must not be in an identifiable format when transmitted between systems.

- Task 5.2

- Reasonable user roles - Who can have access to which information.

- Authorised persons only see the information they need.

- Task 5.1

- Energy Community management - All the necessary information about Energy Communities is available and easily accessible from the customer information system (EnerimCIS).

- There is a field in User Interface to all the needed information and it can be transmitted to third party systems safely.

- Task 5.2

- Unauthorized or Malicious Actuation - Risk of actuation commands being triggered without proper authorization (e.g., bypassing consent rules, spoofing requests from CEL/DSO/Aggregator) or commands being intercepted and replayed/modified.

- Strict Server-Side Authorization Checks: Before issuing any actuation command, the platform must perform rigorous authorization checks, verifying alignment with established business logic, received flexibility requests from authorized partners (CEL, Aggreg ator), and explicit, up-to-date user consent records.

- Contextual Validation: Implement checks to ensure the requested actuation is reasonable within the current context (e.g., safety limits, device state).

- Least Privilege for Actuation Services: Ensure the specific microservices or components responsible for issuing commands only have the necessary permissions to control relevant devices based on validated requests.

- Secure Audit Logging: Log all actuation requests, authorization checks, and command issuance events securely for monitoring and forensic analysis.

- Platform & Fog Node Compromise - Unauthorized access to the central Energy Community Management Platform, Actuation Platform, or local fog nodes could allow attackers to manipulate data, user settings, trigger widespread false actuations, or gain deeper network access.

- System Hardening: Apply security hardening benchmarks and best practices to the operating systems, databases, web servers, and application runtimes on platforms and fog nodes. Minimize the attack surface by disabling unnecessary services and ports.

- Regular Vulnerability Management: Establish processes for continuous vulnerability scanning and timely patching of all software components (OS, libraries, applications). Conduct periodic penetration testing.

- Secure Software Development Lifecycle (SSDLC): Develop platform software following secure coding practices (e.g., mitigating OWASP Top 10 risks) to prevent common vulnerabilities.

- Data availability - Data is available in a timely manner.

- The system (EnerimCIS) supports 15 minutes of energy consumption data and can transmit it to third party systems.

- Task 5.2

- Lack of Security Visibility & Management - Difficulty in monitoring the security posture and detecting threats across a distributed network of IoT devices and fog nodes in real -time. Scalability challenges in managing identities, credentials, and security policies for numerous endpoints.

- Comprehensive and Centralized Logging: Implement detailed security event logging across devices, fog nodes, platforms, and APIs. Aggregate logs securely into a central system (e.g., SIEM) for correlation and analysis, protecting them against tampering.

- Real-time Monitoring and Alerting: Establish continuous monitoring of system health, performance, and security logs. Configure automated alerts for critical security events, anomalies, or policy violations to enable rapid detection.

- Endpoint Security Monitoring: Implement mechanisms for monitoring the security posture of IoT devices and fog nodes, potentially including health checks, configuration drift detection, or lightweight endpoint detection and response (EDR) agents where feasible.

4.3.2 Requirements

- Data storage security for sensitive data

  - Data to be validated and encrypted or otherwise protected within databases

- Database connections everywhere encrypted

  - Database connections are always encrypted, and data is sent in a secure manner.

- Protected data management via REST API calls

  - REST API to support data protection and return only allowed data content through API call

- User Management Improvements

  - User roles and user groups to support detailed access management. New authentication solution with full features into use for product.

- Energy Community Management

  - UI improvements and REST API calls to support Energy Community management. Development of new features and improvements for handling of Energy Communities.

- Data availability

  - 15-minute energy usage data need to be available for the end user on time

- Secure and Authorized Actuation Workflow

  - Actuation commands must be verified for integrity and authenticity. The workflow must ensure commands originate from authorized sources and align with established user consent and operational logic managed by the platform. Implement safeguards against replay attacks.

- Privacy-Preserving Data Handling by Design

  - Platforms and processes must adhere to data minimization principles. Implement secure storage for sensitive data. Provide mechanisms to support granular user consent management. Employ pseudonymization or anonymization techniques where feasible, particularly for analytics.

- Comprehensive Security Logging and Monitoring:

  - Implement detailed security logging across all layers (devices, fog, platform, APIs). Ensure logs are collected centrally, protected from tampering, and integrated with monitoring systems to enable timely threat detection and response. Develop scalable mechanisms for managing device identities, credentials, and security policies.

- Secure API Development and Governance:

  - Develop all APIs according to security best practices (e.g., OWASP API Security Top 10). Implement robust authentication and authorization for API clients (e.g., using OAuth2/OIDC). Apply input validation, output encoding, and rate limiting

D6.1 - Planned 1st year demonstration

Experiments

| UC4_Exp_1 | UC4: 1st year Planned UC4_Exp_1 |
| --- | --- |
| Experiment Name | Data retrieval |
| Aim and Scope | Smart metering data from the field pilot need to be gathered for application of AI algorithms. |
| Methodology | FTP folder is shared by DSO with the energy community manager. ISEP is able to read the information and apply the algorithms. |
| Partner contributions | CEL: Select the field consumers DIGITALMENTE: provide acess to the data ISEP: Obtain knowledge from the data ENERIM: Provide access to consumers data (such as contract data) in customer information management system (EnerimCIS) via REST API interfaces. |
| Expected Results | Confirmation of the data sharing operating well. |

| UC4_Exp_2 | UC4: 1st year Planned UC4_Exp_2 |
| --- | --- |
| Experiment Name | AI algorithms for energy forecast |
| Aim and Scope | Using field data to make energy consumption forecast. |
| Methodology | FTP shared data is tested with AI algorithm for energy consumption forecast. |
| Partner contributions | DIGITALMENTE: provide Access to the data ISEP: Obtain knowledge from the data |
| Expected Results | Confirmation of the data sharing operating well. |

Demonstration

| UC4_Demo_1 | UC4: 1st year Planned UC4_Demo_1 |
| --- | --- |
| Demonstration Name | Energy forecast provision |
| Demonstration description | This demonstration gathers the results of experiments 1 and 2, where it will be seen the collection of data from smart metering, use of AI algorithm for energy forecast, and return to the user. |
| Methodology | The process will work applying the contribution of each partner step by step as detailed below. |
| Partner contributions | CEL: Select the field consumers DIGITALMENTE: provide acess to the data ISEP: Obtain knowledge from the data ENERIM: Provide access to consumers data (such as contract data) in customer information management system (EnerimCIS) via REST API interfaces. DIGITALMENTE: Make results available to consumers. |
| Types of demonstrations (live / prerecorded) | Live with some pre-recoded data. |